# 4-Way Intersection Control with Raspberry Pi 5

## Team Name : Group-4

## Student IDs : 2430745 & 2000000

# Design

The controller treats North/South as one non-conflicting traffic group and the same is true for East/West. The program always starts in an all-red state; then, it follows the sequence :

START/ALL RED → NORTH-SOUTH GREEN → NORTH-SOUTH YELLOW → ALL RED/CLEARANCE → EAST-WEST GREEN → EAST-WEST YELLOW → ALL RED/CLEARANCE → REPEAT.

The code uses helper functions, a LIGHTS dictionary, a continuous loop, safety checking and try/except/finally handling.

# Timing Choices

The assignment states that the GREEN right-of-way phase should last 10 seconds, so the GREEN_TIME constant is set to 10 seconds in the code. The YELLOW phase and the 1-second all-red clearance are additional transitions that last much less time. The all-red interval was selected as a brief safety clearance before the opposite light group turns to green.

# Edge Cases and Safety

| Edge Case | Response |
| --- | --- |
| Power-on / Startup | The system initializes in ALL RED and hold for 2 seconds before normal operation. |
| Conflicting greens | The state sequence never requests both groups green. Additionally, verify_no_conflicting_greens() runs after every transition and, if two conflicting greens are detected, forces a safe state and raises an error. |
| Green → Yellow → Red | Every green phase is followed by its yellow phase. The next opposite green is only enabled after an ALL RED clearance. |
| All-red clearance | A 1-second ALL RED interval is used between the transition of light groups to provide an explicit safety buffer. |
| Program interruption | The KeyboardInterrupt exception is caught and the finally block calls safe_shutdown(), which forces a safe state. |
| Unexpected exception | The program catches any exception using a try/except block. Then, the finally block forces a safe shutdown. |
| Invalid/incorrect pin setup | The program optionally runs a wiring_diagnostic() function that goes through each LED one by one in order to help the user check if they are all connected properly. |
| LED failure | There is no way to check for an LED failure using the software alone, which is something that a real safety-critical controller would need. If an LED was to fail, the system would most likely treat it as an unexpected exception and force a safe shutdown. |
| Timing drift/blocking | The program uses only the intended sleep() intervals in the state sequence. Once the main loop starts, there are no unnecessary delays. |
| Restart after failure | The program executes all_red() at startup, so every restart begins from a known software state. |

# Testing

| Test | Expected Result | Observed Result |
| --- | --- | --- |
| Individual LEDs | Each of 12 LEDs turns on with the correct BCM pin. |  |
| All four red LEDs | N/S/E/W red simultaneously |  |
| North-South group | N/S green together ¦ E/W red |  |
| East-West group | E/W green together ¦ N/S red. |  |
| Three full cycles | The sequence repeats correctly with no conflicting greens |  |
| Ctrl+C during green | Program leaves intersection ALL RED. |  |
| Restart | Program begins ALL RED before first green. |  |

# Troubleshooting

If an LED does not respond, stop the program and check the BCM number, LED orientation, resistor connection, jumper wire and the common ground.

Test one LED at a time before running the complete controller.

NEVER connect an LED directly to a Raspberry Pi GPIO without its current-limiting resistor.

# Submission Checklist

- traffic_intersection.py

- circuit_diagram.png

- state_diagram.png

- report.pdf

- prototype.png

- Video/prototype.mp4

- Zip folder as Group4_2430745_2000000.zip